| S.N. | ACTIVITY | HAZARD | CAUSES OF HAZARD | CONSEQUENCES/ IMPACT | RISK EVALUATION |  | RISK LEVEL | IMPLEMENTED CONTROL MEASURES | RESIDUAL RISK |  |  | ACCEPT Y/N? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P | S | H/M/L |  | P | S | H/M/L |  |
| 1 | Work at height, Scaffolding erection | Improper scaffold material, improper loading, improper handrails, scaffold collapse, material fall, fall from height, heat stress | Improper fall protection, Failure to follow the usage of proper P.P.E Unsafe scaffold condition Material haphazardly stack and strewn on working platform Lack of rehydrate exercise | Fall injury, Fatal, Heat stroke | 4 | 4 | 16 | - Ensure all has HSE orientation - Use proper scaffold and access platform. Ref. EMCO/GN/RA/018 - The periphery of the building is to be barricaded. - Use safety belt. - Do not allow throwing of any material. - Wear safety helmet with chinstrap. Use approved material. Each scaffold platform must have 3 boards at least 228 mm wide and 38 mm thick. Hand and knee rail to be at a height of 900 mm. Deploy certified, competent persons to erect and dismantle scaffolding. Do not keep or store unwanted materials on scaffold platform. Use safety harness. Scaffold should be properly tied to the building. Use PPE, cool water; work in shifts to avoid heat stress. Give proper awareness training. | 1 | 4 | 4 | Y |
| 2 | Plastering on working platform | Fall from heights Tripping and falling Falling objects | Sub standard scaffold structure with missing guard rail ,toe board Unauthorized modification of scaffold members | Fall injury, Fatal, knocks | 3 | 3 | 9 | - Use of proper hand gloves - Training and safety procedures to be followed - Proper scaffolding to be used and safety harness to be used and secured. Use mechanical equipment as far as possible. If mechanical equipment is impractical then use right lifting techniques. Area below the working to be evacuated and warning signs displayed. Sufficient lighting to be provided and placed so as to avoid shadow. Persons to be trained in the correct use of equipment. Area to be kept clean all times | 1 | 3 | 3 | Y |
| 3 | Cement plaster preparation (Mixing)& application | Chemical pollution, Dust pollution, Skin irritation, Eye injury. | Untrained personnel, not wearing correct PPE | Respiratory complication ,occupational illness | 2 | 2 | 4 | - Use hand tools/mechanical means/aid to reduce direct contact with skin. - Use full sleeve cover all and gloves.Toolbox talk, Wearing dust mask, Safety goggles - Body wash facilities, Confined storage of materials, close supervision. Disposal of waste materials on regular intervals. | 1 | 2 | 3 | y |
| 4 | Use of power and hand tools | ### Electrocution, Falling objects, rotating mixer accidents | Untrained personnel, defective tools tools placed haphazardly | Fatal ,bodily injury | 2 | 2 | 4 | - All power tools to be properly earthed, Evacuate the area below the application with warning signs, All rotating wheels to have safety guards All cables to be secured. Tool box talk by certified electrician | 2 | 2 | 4 | Y |
| 5 | ### House keeping & Waste material disposal | ### Untidy site ### Waste accumulation on site | ### Untrained personnel, ### Lack of supervision | ### Inhalation of dust ### Slip and trip hazard ### Personal injury | ### 3 | ### 2 | ### 6 | ### Site to be clean and tidy all the waste resulting from the day’s work to be collected and carted away to the approved dumping area. Also after the completion of the activity all the waste material to be collected and dumped the approved dumping yard. Use good construction practices to minimize the waste and to ensure a better housekeeping. Tool box meeting to be conducted. | 1 | 2 | 2 | Y |

##### Persons in danger

- Workers undertaking the work

- Persons passing the site location/ truck routes, e.g. members of the public and persons in the Vehicles.

##### Personal protective equipment

- Safety Helmet, Gum Boot, Proper Gloves & Safety Goggles

##### Information, instruction and training

- . All personnel in the team are to be made aware of the safe systems of work.

- Ensure that good house keeping standard is maintained through out. - Training and experience for personnel to use electrical equipments and vibratory equipments. - Educate the personnel to handle chemicals such as concrete…

##### Emergency procedures

- Site emergency procedures should make provision for the rescue of individuals from heights.

- First-aid facilities should be available to cope with any significant injuries that may arise from the work.

##### Monitoring and review

- Work should be monitored by Foreman and charge hand who are trained to ensure that any additional precautions or equipment required are provided.

RISK ASSESSMENT COMMITTEE: 		Project Manager/ Project Engineer

Safety Manager/ Safety Officer

Reviewed by:				(Safety Officer)			Approved by:				(Project Manager)

RISK MATRIX

Risk Rating (RR) – Severity x Likelihood

| LIKELIHOOD (PROBABILITY) (P) SEVERITY (IMPACT) (S) |  | Rare Remote possibility (once every 3 years or more) 1 | Unlikely Could happen but rare (typically once in a year) 2 | Possible Could happen occasionally (on average quarterly) 3 | Likely Could happen often (on average once a month or more) 4 | Almost certain Could happen frequently (once a week or more) 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Insignificant | 1 | Low 1 | Low 2 | Low 3 | Low 4 | Medium 5 |
| Minor | 2 | Low 2 | Low 4 | Medium 6 | Medium 8 | Medium 10 |
| Moderate | 3 | Low 3 | Medium 6 | Medium 9 | Medium 12 | High 15 |
| Significant | 4 | Low 4 | Medium 8 | Medium 12 | High 16 | High 20 |
| Major | 5 | Medium 5 | Medium 10 | High 15 | High 20 | High 25 |

RISK BASED CONTROL PLAN

| RISK LEVEL | ACTION AND TIMESCALE |
| --- | --- |
| 1-4 Low | Quick, easy controls should be implemented immediately and further action planned for when resources permit. Monitoring required ensuring controls are maintained. Manage through routine procedures. Go for economic improvements where possible. Incident report must be completed. |
| 5-12 Medium | Aim to reduce risks but costs of prevention may be limited. Undertake a risk assessment of the situation / task and implement the appropriate actions. Actions should have a timescale and should be monitored. Where the risk involves work in progress undertake a risk assessment as soon as possible to ensure the safety of the situation or task. Work should not start until the risk is reduced to an acceptable level. Considerable resources may have to be allocated. Contact your Manager and Risk Manager by telephone about the actions that should be taken to reduce the risk/s. incident report must be completed. Incident must be added to service risk register. |
| 15-25 High | Do not commence the activity until a risk assessment has been completed to ensure the safety of the situation or task. If it is not possible to reduce or eliminate the risk even with unlimited resources, work must remain prohibited. Inform your relevant Director, your Manager and the Risk Manager immediately by telephone. Incident report must be completed. Incident must be added to service risk register. |