| S.N. | ACTIVITY | HAZARD | CAUSES OF HAZARD | CONSEQUENCES/ IMPACT | RISK EVALUATION |  | RISK LEVEL | IMPLEMENTED CONTROL MEASURES | RESIDUAL RISK |  |  | ACCEPT Y/N? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P | S | H/M/L |  | P | S | H/M/L |  |
| 1 | Delivery and mobilization of personnel and equipment | Pinned ,Pinch point (crush) Hit by moving vehicle Falling object | Unsafe and hasty maneuvering of vehicle Narrow access ( space constraint Blind spot Absence of banksman | Fatality, injury to the person and property damage | 3 | 3 | 9 | - Ensure all operators/drivers has HSE orientation - Ensure that the truck roads are to be identified and location to be marked in the drawings - Trained flag man/ signal man to control vehicle maneuvering - Trespassing of personnel on the route is to be monitored - Reverse alarms for trucks is to be operational while reversing - Unauthorized personnel or other workers are to be restricted from trucks movement path. - Safe distance to be maintained between trucks and personnel | 1 | 4 | 4 | Y |
| 2 | Preparation and mixing | Contact to skin and eye Inhalation of fumes and vapors | Untrained personnel performing task Failure to follow correct usage of PPE Hasty action resulting in spillage | Skin irritant Minor Loss of vision Dizziness and respiratory infliction | 3 | 3 | 9 | - Ensure all workers has HSE orientation - Refer MSDS/container labels before using. Use dust masks, goggles, Gloves and full sleeve coverall. (S/P) - Unauthorized personnel or other workers are to be restricted from work area - Portable eyewash bottle shall be available at work location. (S) Use safety goggles and face shields, wherever necessary (S/P) - Ensure good ventilation. Wear suitable respiratory equipment. (Refer MSDS (S) Avoid spillage and always keep limited quantities as demanded by the job. - Wherever required, use mechanical ventilation | 2 | 2 | 4 | y |
| 3 | Application Coating system | Contact to skin and eye Inhalation of fumes and vapors | Untrained personnel performing task Failure to follow correct usage of PPE Hasty action resulting in spillage and accidental splash ,spray | Skin irritant Minor Loss of vision Dizziness and respiratory infliction | 3 | 3 | 9 | - Ensure all workers has HSE orientation - Refer MSDS/container labels before using. Use dust masks, goggles, Gloves and full sleeve coverall. (S/P) - Unauthorized personnel or other workers are to be restricted from work area - Portable eyewash bottle shall be available at work location. (S) Use safety goggles and face shields, wherever necessary (S/P) - Ensure good ventilation. Wear suitable respiratory equipment. (Refer MSDS (S) Avoid spillage and always keep limited quantities as demanded by the job. - Wherever required, use mechanical ventilation | 2 | 2 | 4 | Y |
| 4 | Storage of Coating Materials | Fire hazard | Store near electrical circuitry Naked light ignition by smoking/hot work in close proximity | Fatality, injury Damage to property | 4 | 4 | 16 | - All the personnel involved in the job shall be required to change the dress daily and wash hands thoroughly. - Smoking shall be strictly prohibited at work area - Mixing shall be done only with recommended practice. - Follow precautions as per MSDS - Provide separate storage area. - Provide suitable fire extinguishers in the storage and handling area (S). - Container must be tightly closed. Flammable works shall not be carried out nearby | 1 | 4 | 4 | Y |
| 5 | Housekeeping | Trip ,slip and fall | Indiscriminate placement of materials | Injury to limbs Environmental effect | 3 | 3 | 9 | - Housekeeping must be done on daily basis. All materials - must be kept in one designated area where it does not - create obstruction or potential injuring condition to site - personnel. Storage area must be provided and identified with adequate ventilation - so that all superfluous materials are properly kept - and transported from within the construction site (Area) - Provision of proper disposal bin and waste refuse provider | 1 | 2 | 2 | y |

##### Persons in danger

- Workers undertaking the work

- Persons passing the site location/ truck routes, e.g. members of the public and persons in the Vehicles.

##### Personal protective equipment

- Safety Helmet, Gum Boot, Proper Gloves & Safety Goggles

##### Information, instruction and training

- All personnel in the team are to be made aware of the safe systems of work.

- Ensure that good house keeping standard is maintained through out. - Training and experience for personnel to use electrical equipments and vibratory equipments. - Educate the personnel to handle chemicals such as concrete…

##### Emergency procedures

- Site emergency procedures should make provision for the rescue of individuals from heights.

- First-aid facilities should be available to cope with any significant injuries that may arise from the work.

##### Monitoring and review

- Work should be monitored by Foreman and charge hand who are trained to ensure that any additional precautions or equipment required are provided.

RISK ASSESSMENT COMMITTEE: 		Project Manager/ Construction Manager/ Project Engineer

Safety Manager/ Safety Officer

Reviewed by:				(Safety Officer)			Approved by:				(Project Manager)

RISK MATRIX

Risk Rating (RR) – Severity x Likelihood

| LIKELIHOOD (PROBABILITY) (P) SEVERITY (IMPACT) (S) |  | Rare Remote possibility (once every 3 years or more) 1 | Unlikely Could happen but rare (typically once in a year) 2 | Possible Could happen occasionally (on average quarterly) 3 | Likely Could happen often (on average once a month or more) 4 | Almost certain Could happen frequently (once a week or more) 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Insignificant | 1 | Low 1 | Low 2 | Low 3 | Low 4 | Medium 5 |
| Minor | 2 | Low 2 | Low 4 | Medium 6 | Medium 8 | Medium 10 |
| Moderate | 3 | Low 3 | Medium 6 | Medium 9 | Medium 12 | High 15 |
| Significant | 4 | Low 4 | Medium 8 | Medium 12 | High 16 | High 20 |
| Major | 5 | Medium 5 | Medium 10 | High 15 | High 20 | High 25 |

RISK BASED CONTROL PLAN

| RISK LEVEL | ACTION AND TIMESCALE |
| --- | --- |
| 1-4 Low | Quick, easy controls should be implemented immediately and further action planned for when resources permit. Monitoring required ensuring controls are maintained. Manage through routine procedures. Go for economic improvements where possible. Incident report must be completed. |
| 5-12 Medium | Aim to reduce risks but costs of prevention may be limited. Undertake a risk assessment of the situation / task and implement the appropriate actions. Actions should have a timescale and should be monitored. Where the risk involves work in progress undertake a risk assessment as soon as possible to ensure the safety of the situation or task. Work should not start until the risk is reduced to an acceptable level. Considerable resources may have to be allocated. Contact your Manager and Risk Manager by telephone about the actions that should be taken to reduce the risk/s. incident report must be completed. Incident must be added to service risk register. |
| 15-25 High | Do not commence the activity until a risk assessment has been completed to ensure the safety of the situation or task. If it is not possible to reduce or eliminate the risk even with unlimited resources, work must remain prohibited. Inform your relevant Director, your Manager and the Risk Manager immediately by telephone. Incident report must be completed. Incident must be added to service risk register. |