| S.N. | ACTIVITY | HAZARD | CAUSES OF HAZARD | CONSEQUENCES/ IMPACT | RISK EVALUATION |  | RISK LEVEL | IMPLEMENTED CONTROL MEASURES | RESIDUAL RISK |  |  | ACCEPT Y/N? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P | S | H/M/L |  | P | S | H/M/L |  |
| 1 | Delivery and storage of material at site | Chemical, spillage | Improper handling Exposure to uncovered chemical | Side effect illness, Damaged to property | 2 | 2 | 4 | - Ensure that materials are stored separately with identification and MSDS posted and displayed significantly - Provide fire extinguisher at storage area including signages. | 1 | 1 | 3 | y |
| 2 | Application of material | Chemical, spillage Inhalation of chemical Eye injuries and irritation | Improper handling Exposure to uncovered chemical | Side effect illness, Damaged to property | 2 | 3 | 6 | - Ensure that personnel assigned are trained and experienced in the trade. - Ensure that all personnel are wearing safety gloves (long sleeve), mask, and other basic PPE as per the instructions in the relevant MSDS | 2 | 2 | 4 | y |
| 3 | Housekeeping & disposal of waste | Environmental hazard | Poor house keeping Lack of interest towards site cleaning | Soil contamination Air contamination | 2 | 2 | 4 | - Provide second containment pad / drip tray - Ensure that MSDS is understood by the site operatives and readily available at site. - Conduct tool box meeting before starting the activity and describe the hazards and control measures to the operatives. | 1 | 2 | 2 | y |

##### Persons in danger

- Workers undertaking the work

- Persons passing the site location/ truck routes, e.g. members of the public and persons in the Vehicles.

##### Personal protective equipment

- Safety Helmet, Gum Boot, Proper Gloves & Safety Goggles

##### Information, instruction and training

- All personnel in the team are to be made aware of the safe systems of work.

- Ensure that good house keeping standard is maintained through out. - Training and experience for personnel to use electrical equipments and vibratory equipments. - Educate the personnel to handle chemicals such as concrete…

##### Emergency procedures

- Site emergency procedures should make provision for the rescue of individuals from heights.

- First-aid facilities should be available to cope with any significant injuries that may arise from the work.

##### Monitoring and review

- Work should be monitored by Foreman and charge hand who are trained to ensure that any additional precautions or equipment required are provided.

RISK ASSESSMENT COMMITTEE: 		Project Manager/ Project Engineer

Safety Manager/ Safety Officer

Reviewed by:				(Safety Officer)			Approved by:				(Project Manager)

RISK MATRIX

Risk Rating (RR) – Severity x Likelihood

| LIKELIHOOD (PROBABILITY) (P) SEVERITY (IMPACT) (S) |  | Rare Remote possibility (once every 3 years or more) 1 | Unlikely Could happen but rare (typically once in a year) 2 | Possible Could happen occasionally (on average quarterly) 3 | Likely Could happen often (on average once a month or more) 4 | Almost certain Could happen frequently (once a week or more) 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Insignificant | 1 | Low 1 | Low 2 | Low 3 | Low 4 | Medium 5 |
| Minor | 2 | Low 2 | Low 4 | Medium 6 | Medium 8 | Medium 10 |
| Moderate | 3 | Low 3 | Medium 6 | Medium 9 | Medium 12 | High 15 |
| Significant | 4 | Low 4 | Medium 8 | Medium 12 | High 16 | High 20 |
| Major | 5 | Medium 5 | Medium 10 | High 15 | High 20 | High 25 |

RISK BASED CONTROL PLAN

| RISK LEVEL | ACTION AND TIMESCALE |
| --- | --- |
| 1-4 Low | Quick, easy controls should be implemented immediately and further action planned for when resources permit. Monitoring required ensuring controls are maintained. Manage through routine procedures. Go for economic improvements where possible. Incident report must be completed. |
| 5-12 Medium | Aim to reduce risks but costs of prevention may be limited. Undertake a risk assessment of the situation / task and implement the appropriate actions. Actions should have a timescale and should be monitored. Where the risk involves work in progress undertake a risk assessment as soon as possible to ensure the safety of the situation or task. Work should not start until the risk is reduced to an acceptable level. Considerable resources may have to be allocated. Contact your Manager and Risk Manager by telephone about the actions that should be taken to reduce the risk/s. incident report must be completed. Incident must be added to service risk register. |
| 15-25 High | Do not commence the activity until a risk assessment has been completed to ensure the safety of the situation or task. If it is not possible to reduce or eliminate the risk even with unlimited resources, work must remain prohibited. Inform your relevant Director, your Manager and the Risk Manager immediately by telephone. Incident report must be completed. Incident must be added to service risk register. |