| S.N. | ACTIVITY | HAZARD | CAUSES OF HAZARD | CONSEQUENCES/ IMPACT | RISK EVALUATION |  | RISK LEVEL | IMPLEMENTED CONTROL MEASURES | RESIDUAL RISK |  |  | ACCEPT Y/N? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P | S | H/M/L |  | P | S | H/M/L |  |
| 1 | Delivery and mobilization of personnel and equipment | JCB dual function (bucket/hammer) | Unsafe maneuvering of JCB | Fatality, injury to the person and property damage, accident between JCB workers involved on task | 4 | 4 | 16 | - Ensure all operators/drivers has HSE orientation - Ensure that the truck roads are to be identified and location of JCB to be marked in the drawings - Trained flag man/ signal man to control vehicle maneuvering - Trespassing of personnel on the route is to be monitored - Reverse alarms for trucks is to be operational while reversing - Unauthorized personnel or other workers are to be restricted from trucks/JCB movement path. - Safe distance to be maintained between trucks/JCB. | 1 | 4 | 4 | Y |
| 2 | Cutting & hacking of concrete wall/flloor | Miss handling of exposed rebar/brc during cutting and removal | Failure to follow the usage of proper P.P.E | cuts, injury due to chemicals | 4 | 3 | 12 | - Use of proper hand gloves - Training and safety procedures to be followed | 1 | 3 | 3 | Y |
| 3 | Pumping concrete | Equipment, Failure of pumps | Improper inspection of joints and unskilled operators and mechanical failure | Fatality, injury | 3 | 4 | 12 | - Ensure spill precautions by putting polythene sheets and timely removal to designated place - Proper clamping of hose pipe of pump - Competent Operator License - Ensure pump has valid certificate | 1 | 4 | 4 | Y |
| 4 | Pouring concrete with bucket | Failure of concrete bucket | 2 Load can fall (Concrete) | Severe personal injury, Property damage | 4 | 5 | 20 | - Make sure competent operator and rigger are assigned - Use certified equipment / lifting tackles - Use third party certified buckets - Use tag line - Follow the SWL - Inspect the bucket during use - Cordon off the area - Close supervision | 1 | 5 | 5 | Y |
| 5 | Placing concrete on columns& slabs | Miss handling of concrete hose | Failure to deploy trained personnel and barricading the area while cleaning process in progress, working at height | Fatality, injury | 3 | 4 | 12 | - The personnel handling the hose pipe is to be trained and use of higher diameter nylon rope as for guiding the hose while concreting - Controlling and cleaning of pump - Use of full body harness | 1 | 4 | 4 | Y |
| 6 | While placing concrete | Slips and trips | Improper housekeeping and negligence of the personnel | Fracture, penetrating or puncture injury | 3 | 4 | 12 | - House keeping to be maintained - Re-enforcement placing top and bottom bars are to be tied properly to avoid unbalance while personnel walking over the steel | 1 | 4 | 4 | Y |
| 7 | While placing concrete | Fall of personnel from height | Failure to follow the rules on working at heights | Fatality, injury of person | 4 | 4 | 16 | - Use appropriate working plate form with abdicate fall protection system - Ensure that the edge protection handrails system of the shuttering is properly fixed and in place. - Use job specific PPE like, ie: goggle, hand gloves, gum shoes and mask | 1 | 4 | 4 | Y |
| 8 | While placing concrete | Collapse of formwork | Failure in the structural design with respect to the load calculations Failure in provision of back propping system as per design standards | Fatality, injury of the person and property damage | 3 | 5 | 15 | - Ensure the formwork supplier is to assess the supporting system is adequate according to the design. Certification to be given to Main Contractor prior to the Consultant inspection for concerting. - Ensure a carpenter team must be made available while casting to check the back propping system is not disturbed/ altered | 1 | 5 | 5 | Y |
| 9 | Use of vibrator | Repeated vibratory equipment use | No proper training in usage of vibrator | Carporal syndrome (white finger disease) for personnel | 2 | 4 | 8 | - Work cycle method / rotation system in usage of vibrator for placing of concrete - Control repetition work | 1 | 4 | 4 | Y |
| 10 | Usage of electrical machineries | Unsafe use of electrical machineries | No proper maintenance of electrical equipments/ machineries Failure to deploy skilled electricians | Electrocution of personnel | 3 | 4 | 12 | - Periodic check of electrical equipments/ machineries - Electrical connection of power supply/ cable management and arrangements | 1 | 4 | 4 | Y |
| 11 | Concreting on day | Sunstroke | Failure to follow the local and ministerial regulations | Sunburn Collapse of personnel | 3 | 4 | 12 | - Adequate drinking water supply along with dehydration supplement like ORS… - First Aid facility is made available throughout | 1 | 4 | 4 | Y |
| 12 | Concreting at night | Slips & trips | Failure in providing proper illumination | Personnel injury, fracture | 4 | 4 | 16 | - Proper lighting facilities provision - Approval / NOC for night concreting - Implement HSE concrete pouring checklist | 1 | 4 | 4 | Y |
| 13 | Scaffold erection for working platform | Scaffold topple | Unstable footing and lnadequate outrigger/tie-back support | Injury to workman | 4 | 4 | 16 | - Scaffold should be erected on a level ground - No scaffold construction is allowed on or near any excavated areas - Scaffold should be erected only by the approved Scaffold erectors and certified by Scaffold Supervisor - Scaffold Materials should be free from defect - When scaffold erection is more than 4m it should erected by the approved scaffold contractors - When ever metal scaffold height is more than 30m it should be designed by the Professional Engineer (PE) - When the scaffold requires cantilever support or erected on the temporary support it should designed by the PE. - Mobile scaffold shall not exceed 3times it’s least base width without any tie. - Mobil scaffold height shall not exceed 8times its least base width with tie. - Do not remove any ties or props without scaffold supervisor approval. Scaffold supervisor shall ensure scaffold is erected as per the scaffold regulations | 1 | 4 | 4 | y |

##### Persons in danger

- Workers undertaking the work

- Persons passing the site location/ truck routes, e.g. members of the public and persons in the Vehicles.

##### Personal protective equipment

- Safety Helmet, Gum Boot, Proper Gloves & Safety Goggles

##### Information, instruction and training

- All personnel in the team are to be made aware of the safe systems of work.

- Ensure that good house keeping standard is maintained through out. - Training and experience for personnel to use electrical equipments and vibratory equipments. - Educate the personnel to handle chemicals such as concrete…

##### Emergency procedures

- Site emergency procedures should make provision for the rescue of individuals from heights.

- First-aid facilities should be available to cope with any significant injuries that may arise from the work.

##### Monitoring and review

- Work should be monitored by Foreman and charge hand who are trained to ensure that any additional precautions or equipment required are provided.

RISK ASSESSMENT COMMITTEE: 		Project Manager/ Project Engineer

Safety Manager/ Safety Officer

Reviewed by:				(Safety Officer)			Approved by:				(Project Manager)

RISK MATRIX

Risk Rating (RR) – Severity x Likelihood

| LIKELIHOOD (PROBABILITY) (P) SEVERITY (IMPACT) (S) |  | Rare Remote possibility (once every 3 years or more) 1 | Unlikely Could happen but rare (typically once in a year) 2 | Possible Could happen occasionally (on average quarterly) 3 | Likely Could happen often (on average once a month or more) 4 | Almost certain Could happen frequently (once a week or more) 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Insignificant | 1 | Low 1 | Low 2 | Low 3 | Low 4 | Medium 5 |
| Minor | 2 | Low 2 | Low 4 | Medium 6 | Medium 8 | Medium 10 |
| Moderate | 3 | Low 3 | Medium 6 | Medium 9 | Medium 12 | High 15 |
| Significant | 4 | Low 4 | Medium 8 | Medium 12 | High 16 | High 20 |
| Major | 5 | Medium 5 | Medium 10 | High 15 | High 20 | High 25 |

RISK BASED CONTROL PLAN

| RISK LEVEL | ACTION AND TIMESCALE |
| --- | --- |
| 1-4 Low | Quick, easy controls should be implemented immediately and further action planned for when resources permit. Monitoring required ensuring controls are maintained. Manage through routine procedures. Go for economic improvements where possible. Incident report must be completed. |
| 5-12 Medium | Aim to reduce risks but costs of prevention may be limited. Undertake a risk assessment of the situation / task and implement the appropriate actions. Actions should have a timescale and should be monitored. Where the risk involves work in progress undertake a risk assessment as soon as possible to ensure the safety of the situation or task. Work should not start until the risk is reduced to an acceptable level. Considerable resources may have to be allocated. Contact your Manager and Risk Manager by telephone about the actions that should be taken to reduce the risk/s. incident report must be completed. Incident must be added to service risk register. |
| 15-25 High | Do not commence the activity until a risk assessment has been completed to ensure the safety of the situation or task. If it is not possible to reduce or eliminate the risk even with unlimited resources, work must remain prohibited. Inform your relevant Director, your Manager and the Risk Manager immediately by telephone. Incident report must be completed. Incident must be added to service risk register. |