| Serial No. | Sub-Activities | Hazards Identified (Generic & Task) | Risks Involved (People & Property) | Risk Rating |  |  | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Severity | Probability | Initial Risk |  |  |
| 1. | Prepare work area | · Unauthorized, untrained workers | · Cuts · Abrasions | 2 | A | L | · TBT/DSTI shall be conducted by the foreman/ safety officer prior to work commencement. · Task specific safety training to be provided. · Authorized personnel to undertake the job. · Daily safety inspection should be carried out and eliminate the hazard. | Low (ALARP) |
| 2. | Manual Handling | · Incorrect lifting of loads · Sharp edges. · Placing the object while fingers underneath. • Slip/trip/fall ( same level ) • Unsafe posture | · Back Injuries · Cut to finger · Strain • Musculoskeletal Injuries | 3 | C | M | · Any lifting tasks shall be carried out by persons those physically capable to do so •No employee should be asked to carry loads above his capacity and in any case no load shall exceed 30kg per man. • Mass of the load is more then to be equally shared · Deploy enough number of personnel for lifting depending upon the type of load to be lifted. ·Use mechanical aids such as trolley etc. • Load not to be lifted above your shoulder height • Keep your fingers away from pinch point while placing the load. • Materials shall not be carried through Pedestrian access. • Area shall be illuminated with adequate lighting. · Appropriate personal protective equipments shall be used while manual handling of materials and loads. | Low (ALARP) |

| Serial No. | Sub-Activities | Hazards Identified (Generic & Task) | Risks Involved (People & Property) | Risk Rating |  |  | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Severity | Probability | Initial Risk |  |  |
| 3. | Unloading and shifting of bus bar unit with using forklift or crane | • Unauthorized, Untrained Workers • Lifting failure. • Unsafe lifting. • Swinging of material • Damaged sling/wire rope or a chain • Poor maintenance of vehicle • Speedy operation • Over turn • Over load • Poor communication • Poor lighting | • Fatality • Fractures • Property damage • Injury to personal • Sprain and strain • Crush • Cuts | 4 | C | H | •Pre check to be carried out by the operator prior to start the work to ensure that no hydraulic leaks, or other visible damages seen. • Ensure the crane and forklift engaged is tested and having valid license and the operators has valid license and approved 3rd party certificate. • Engage trained and third party certified riggers shall be provided. • Barrication and warning signs shall be provided for the area where lifting operation is carried out to restrict the entry. • Check list shall be maintained for the vehicles prior to start any work • Use tagline for control of swing and alignment. • All lifting equipments and gears shall have valid third party certificate. • The crane must be stopped when the wind speed exceeds 38km/hr. • Never exceed the SWL marked on the crane. • All load shall be secured prior to lifting. • The crane and trucks inspection shall be conducted periodically and record maintained. ·Vehicle not to be left unattended at site (ie,to be switch off mode if required and keys removed) • Operators and rigger must wear reflecting vest and adequate PPE ( Shoe, helmet, leather gloves, goggles, over all ) ·Vehicle routes kept free of obstructions (with spillages being cleaned up promptly) • Operator and rigger shall be known on the rigging procedure. • Adequate lighting shall be provided for the work location. | M (ALARP) |

| Serial No. | Sub-Activities | Hazards Identified (Generic & Task) | Risks Involved (People & Property) | Risk Rating |  |  | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Severity | Probability | Initial Risk |  |  |
|  |  |  |  |  |  |  | ·Any reversing that is necessary should be overseen by a trained flagman / Banks man and reverse beeper to be fitted to all equipment’s to alert the people around the vicinity. · Operator should follow the site speed limit and obey the traffic rules. • Colour coded system to be implemented to all lifting equipment’s as per the month. • Close supervision to be done at all times. |  |
| 4. | Stacking and Storing | ∙ Unauthorized, untrained workers · Material fall from height • Improper stacking • Collapse of materials | • Crush · Cut and abrasion · Damage to property | 3 | B | M | · All materials shall be stored below 2 meters height. · If the material is too heavy, use hydraulic trolley to shift the material. • No materials shall be stored at work place, only required quantity to be taken for the job. • Materials shall be covered to protect from weather and contaminants. • Personnel protective equipment must be worn at all time during the activity. (Helmet, shoe, vest, coverall, gloves, goggles, mask.) | Low (ALARP) |
| 5. | Installation of bus bar unit | • Untrained, unauthorized Workers • Material Fall • Sharp edges • Unsafe posture • Slip, trip and fall | • Cut and abrasion • Back Injuries • Fatigue • Musculoskeletal Injuries | 3 | C | M | · Competent team shall be involved to execute the activities. ∙Work location shall be inspected by concerned supervisor / foreman. • Close supervision to be done at all times • Materials should not kept in walkways, edge of the opening. · Deploy enough number of personnel for material lifting shall be appointed to the task depending upon the distance • Manufacturer recommendations shall be followed during the installation. | Low (ALARP) |

| Serial No. | Sub-Activities | Hazards Identified (Generic & Task) | Risks Involved (People & Property) | Risk Rating |  |  | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Severity | Probability | Initial Risk |  |  |
|  |  |  |  |  |  |  | • Proper communication to be maintained between the workers while doing the installation • Maintain a good housekeeping policy to prevent from slip, trip, and fall hazards. • Safe and obstruction free access for working personnel’s shall be maintained at all times. • Emergency procedure shall be briefed to all personnel prior to start the activity. • Emergency evacuation plan shall be readily available according to site condition and briefed to the workforce prior to start. • Trained first aider, fire warden and emergency vehicle to be available at the site during the activity. • Emergency contact numbers shall be displayed at work location for easy communication. • PPE to be worn at all times( Gloves, goggles, mask, helmet, shoe, vest, overall) |  |
| 6. | Use of Hand tools ( Torque wrench, spanner, nail puller, cutter ) | • Slip, trip and fall • Damaged / worn out hand tools • Flying objects ∙ Mushroom Heads • Using incorrect type | • Fatigue • Trigger finger • Cuts and abrasions | 3 | C | M | • Defective tools should not be issued or used to perform the task, to be checked by store keeper • Don’t extend the spanner by using a pipe as an extension bar •Hand tools shall be free from oil, grease & etc. • Chisel’s head shall have safety cap to avoid finger injury. • Burs of trunking shall be filed off immediately after cutting. • Never use a file as a lever • Never use a screw driver as a chisel • Use eye/ face protection when there is flying materials hazard, wear gloves while required to protect the hands • Use right tools for the job | Low (ALARP) |

| Serial No. | Sub-Activities | Hazards Identified (Generic & Task) | Risks Involved (People & Property) | Risk Rating |  |  | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Severity | Probability | Initial Risk |  |  |
| 7. | Use of power tools ( drill machine ) | ● Defective equipment - Electricity - Improper use of equipment - Poor maintenance - Untrained workers - Flying object - Noise and vibration - Fire - Dust | - Hearing losses - Eye injury - Crushing - Back injury - Burns - Electrocution - Ringing in the ear / Deafness | 3 | C | M | • PTW shall be applied and obtained prior to start, If spark producing activity is performed · Defective tools should not be issued or used to perform a task. • Operators must be having their training card. • Monthly inspection shall be carried out and tagged out by the electrician and record to be kept at the store • If required, noise measurements shall be carried out. • Use alternative, less noisy tools, equipments and methods. · All portable power tools should be fitted with adequate fuse protection either in the tool body or in its electrical plug. · The weight, size and type of tool should be selected to suit the job being carried out. • Don’t lay the cables around the access way, it should be routed properly. • Industrial distribution board and socket only shall be used at site. • No joints shall be made in electrical cable. • Cable hangers shall be used to avoid the direct contact of power tools cable with the scaffolding members. • Electrical connection shall be routed through ELCB only ( Eg. 30ma ) · Tools should be stored and maintained in good working condition. · Appropriate PPE Shall be used ( Goggles, Mask, Ear plug, Gloves, Shoe, Helmet, Overall, Vest ) | M (ALARP) |

| Serial No. | Sub-Activities | Hazards Identified (Generic & Task) | Risks Involved (People & Property) | Risk Rating |  |  | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Severity | Probability | Initial Risk |  |  |
| 8. | Use of ladder/ scaffold | • Person/ material falling from height • Over reaching • Over loading • Obstructing the access way • Slippery or uneven surface • Overhead structure and cable • Untrained, unauthorized Workers • Collapse of scaffold | • Sprain and strain • Fracture • Spinal cord damage • Musculoskeletal Injury • Fatality | 3 | C | M | • All scaffoldings shall be erected and maintained as per ADM regulations. • Visual inspection should be carried out before use (i.e., hand rails, toe board, on the platform and any opening are there). • Only trained scaffolders shall be involved in erection & dismantling activities. • The ladder should extend minimum one meter from the top for safe movement • Do not erect in slippery, uneven surface • Should be set up with a 4 vertical to 1 horizontal slope ( 4:1 ) • Step ladder must fully open and lock the devices as per manufacturer recommendations. • Do not work from the top of two rungs of step ladders. • Remember the 3 point contact. • Ladder shall be used for short duration works Only limited to 15 min. • Ladder used for access must be firmly fixed or one person must hold for climbing up and down. • Colour code system shall be implemented. • Do not stack materials on to the scaffold. • Keep walkways free of obstacles, tools and equipment. • Work at height is restricted during adverse weather condition. • Adequate lighting shall be provided while working at night. • Safety harness shall be used above 2mtr height and anchored in above shoulder height at all time. | Low (ALARP) |

| Serial No. | Sub-Activities | Hazards Identified (Generic & Task) | Risks Involved (People & Property) | Risk Rating |  |  | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Severity | Probability | Initial Risk |  |  |
| 9. | While working at night shift | ∙ Poor illumination · Slip/trip/fall | ∙ Fatality • Cuts | 3 | B | M | · Work notification shall be applied and obtained prior to start the night work from main contractor. · Sufficient illumination shall be provided. · Light post shall be secured to avoid fall. · Proper cable management system shall be followed · All materials shall be staged in proper manner without obstacles | Low (ALARP) |
| 10. | Working in hot weather | ∙ Heat Stress | ∙ Dehydration · Sunburn · Skin Cancer | 2 | B | M | · Drink plenty of water ∙ Employees shall be trained about heat stress symptoms & first aid measures. · Avoid highly physical tasks during the hottest time of the day. · Isotonic drinks shall be provided. • Provide sufficient rest breaks based on the humidity & temperature conditions. • Heat stress management plan shall be developed and implemented during the activity. | Low (ALARP) |
| 11. | Work Completion | · Poor housekeeping · Slip/trip/fall | · Fire | 2 | A | L | · General housekeeping, Remove all surplus and unwanted waste materials from the work place. · All necessary precautions will be adopted to prevent fire | Low (ALARP) |

RISK MATRIX

|  |  |  |  |  | Probability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | A | B | C |  | D | E |
| Severity | People | Assets | Environment | Reputation | Improbable 1 in 100,000 Years | Remote 1 in 10,000 Years | Occasional 1 in 1000 years |  | Probable 1 in 100 years | Frequent 1 in 10 years |
| 5-Catatrophic | Multiple fatalities or permanent total disabilities | Extensive damage | Massive effect | International impact |  |  |  |  |  |  |
| 4-Severe | Single fatalities or permanent total disabilities | Major damage | Major effect | National impact |  |  |  | High Risk |  |  |
| 3-critical | Major injury or health effects | Local damage | Localised effect | Considerable impact |  | Medium Risk (ALARP) |  |  |  |  |
| 2-Marginal | Minor injury or health effects | Minor damage | Minor effect | Minor impact |  |  |  |  |  |  |
| 1-Negligble | Slight injury or health effects | Slight Damage | Slight effect | Slight impact |  | Low Risk |  |  |  |  |

| HIGH | 3-E 4-C,D,E 5-B,C,D,E |
| --- | --- |
| MEDIUM | 1-D,E 2-B,C,D,E 3-A,B,C,D 4-A,B 5-A |
| LOW | 1-A,B,C 2A |