| S.N. | ACTIVITY | HAZARD | CAUSES OF HAZARD | CONSEQUENCES/ IMPACT | RISK EVALUATION |  | RISK LEVEL | IMPLEMENTED CONTROL MEASURES | RESIDUAL RISK |  |  | ACCEPT Y/N? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P | S | H/M/L |  | P | S | H/M/L |  |
| 1 | Material delivery | Falling object from Loose materials | Unstable stacks; uneven grounds; stacking marble or granite too high so that they may collapse | Injuries to parts of the body; Damage to property (broken marble) | 2 | 2 | 4 | - Off-loading of materials with the use of mobile plant (e.g. forklift); only competent person to operate machine, riggers must be assisting the operator at all times during operation. - Supervisor should have a stacking program at all times indicating direction and height - Floors/grounds used for stacking must be kept level and free from obstructions at all times - All stackers to be trained on how to stack properly and safely - Daily Safe Task Instructions must be conducted by the SRS prior to work commencement on a daily basis indicating all the associated hazards and risks together with the control measures identified under the scope of work. Site operatives must sign in the form as proof of attendance and understanding of all the items raised and discussed | 2 | 2 | 4 | y |
| 2 | Shifting marble and mortar | - Falling Objects. - Failure of Lifting. - Equipment and Gears. -Manual Handling | -Untrained personnel performing task Defective equipment’s | - Injury - Damage to Property | 3 | 3 | 9 | - Use only tested and certified lifting equipment’s and gears. - Use designed and certified unloading platforms. - Engage experienced, trained and certified operator and banks man. - Marble tiles and mortar should be transferred in specified trays and buckets. So far as is reasonably practicable, manual handling which could cause injury should be avoided Where unavoidable, training in appropriate techniques to be undertaken. Where reasonably practicable, provide mechanical aids as appropriate (Sack Barrows, trolleys and other mechanical means. Provide training in the use of mechanical handling equipment. Trolleys should never be use in ramp areas. workplace/station avoiding unnecessary double handling. - . | 2 | 2 | 4 | y |
| 3 | -Applying adhesive - Cement mortar preparation (mixing and application | Dust Pollution - Chemical Exposure | Inadequate PPE provision Failure to deploy trained personnel and barricading the work area | - Illness - Skin irritation - Breathing difficulty - Long term lung cancer | 2 | 2 | 4 | - Provide regular toolbox talk. - Follow the MSDS provided by the manufacturer’s instruction. - Wear air filter dusk mask, Safety goggles. - Provide body wash facilities. - Closed supervision. - Disposal of waste materials on regular intervals. - All chemicals must be supplied with Materials Safety Data Sheets (MSDS) detailing action in an emergency. Data sheets must be submitted to CRM department. - The workplace area must be well ventilated and illuminated to prevent fumes build-up - MSDS should be well communicated to all operatives and instruction should be followed. |  |  |  | Y |
| 4 | Laying of marble tiles, use of grouting material | - Chemical Exposure, Slips, Trips and Fall | Untrained personnel performing task Materials lying haphazardly | - Illness - Skin irritation - Breathing difficulty | 1 | 2 | 2 | - Use MSDS recommended PPE. - Provide hand washing facility. - All personnel must be trained on the use of material. - Restrict the area of execution. - Post warning sign. - All chemicals must be supplied with a Materials Safety Data Sheets detailing action in an emergency and data sheets must be submitted to CRM department - Vigilance to avoid slip and trip hazards caused by discarded equipment, wet floors, cable runs, water hose etc. | 1 | 2 | 2 | Y |
| 5 | Drilling in concrete and fixing of marble or granite | . Persons falling while working at elevated level. | Failure to follow the usage of proper P.P.E Unauthorized alteration of scaffold structure components | Fatality, injury to the person and property damage, accident between | 4 | 4 | 16 | - Use proper scaffold and access platform. - Use Safety belt. (S) - Use safety shoe. (S) - Inspect the ladder/scaffold structure for defects before starting the job. (Use scaffolding inspection tags certify safe to use) - Barrication / caution board. (Caution – Men working overhead) | 2 | 2 | 4 | Y |
| 5 | DRILLING in concrete in case of dry fix | Inhalation of dust | Failure to follow the usage of proper P.P.E | DIFFICULTY FOR BREATHING | 2 | 3 | 6 | - Use dust masks. (S) - Periodical housekeeping to be maintained. | 2 | 2 | 4 | Y |
| 6 | DRILLING in concrete in case of dry fix | Usage of handling of portable power tools. • | Improper inspection of tools and unskilled operators and mechanical failure | Fatality, injury | 3 | 3 | 9 | - – Usage of portable power tools | 2 | 2 | 4 | Y |
| 7 | DRILLING in concrete in case of dry fix | Eye injury | Failure to follow the usage of proper P.P.E Improper inspection of tools and unskilled operators | injury | 4 | 4 | 16 | - Use safety goggles wherever there is a possibility of eye injury (S) - Maintain safe distance with other worker, while doing drilling, chipping operation | 3 | 3 | 9 | y |

##### Persons in danger

- Workers undertaking the work

- Persons passing the site location/ truck routes, e.g. members of the public and persons in the Vehicles.

##### Personal protective equipment

- Safety Helmet, Gum Boot, Proper Gloves & Safety Goggles

##### Information, instruction and training

- All personnel in the team are to be made aware of the safe systems of work.

- Ensure that good housekeeping standard is maintained throughout. - Training and experience for personnel to use electrical equipment’s and vibratory equipment’s. - Educate the personnel to handle chemicals such as concrete…

##### Emergency procedures

- Site emergency procedures should make provision for the rescue of individuals from heights.

- First-aid facilities should be available to cope with any significant injuries that may arise from the work.

##### Monitoring and review

- Work should be monitored by Foreman and charge hand who are trained to ensure that any additional precautions or equipment required are provided.

RISK ASSESSMENT COMMITTEE: 		Project Manager/ Project Engineer

Safety Manager/ Safety Officer

Reviewed by:				(Safety Officer)			Approved by:				(Project Manager)

RISK MATRIX

Risk Rating (RR) – Severity x Likelihood

| LIKELIHOOD (PROBABILITY) (P) SEVERITY (IMPACT) (S) |  | Rare Remote possibility (once every 3 years or more) 1 | Unlikely Could happen but rare (typically once in a year) 2 | Possible Could happen occasionally (on average quarterly) 3 | Likely Could happen often (on average once a month or more) 4 | Almost certain Could happen frequently (once a week or more) 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Insignificant | 1 | Low 1 | Low 2 | Low 3 | Low 4 | Medium 5 |
| Minor | 2 | Low 2 | Low 4 | Medium 6 | Medium 8 | Medium 10 |
| Moderate | 3 | Low 3 | Medium 6 | Medium 9 | Medium 12 | High 15 |
| Significant | 4 | Low 4 | Medium 8 | Medium 12 | High 16 | High 20 |
| Major | 5 | Medium 5 | Medium 10 | High 15 | High 20 | High 25 |

RISK BASED CONTROL PLAN

| RISK LEVEL | ACTION AND TIMESCALE |
| --- | --- |
| 1-4 Low | Quick, easy controls should be implemented immediately and further action planned for when resources permit. Monitoring required ensuring controls are maintained. Manage through routine procedures. Go for economic improvements where possible. Incident report must be completed. |
| 5-12 Medium | Aim to reduce risks but costs of prevention may be limited. Undertake a risk assessment of the situation / task and implement the appropriate actions. Actions should have a timescale and should be monitored. Where the risk involves work in progress undertake a risk assessment as soon as possible to ensure the safety of the situation or task. Work should not start until the risk is reduced to an acceptable level. Considerable resources may have to be allocated. Contact your Manager and Risk Manager by telephone about the actions that should be taken to reduce the risk/s. incident report must be completed. Incident must be added to service risk register. |
| 15-25 High | Do not commence the activity until a risk assessment has been completed to ensure the safety of the situation or task. If it is not possible to reduce or eliminate the risk even with unlimited resources, work must remain prohibited. Inform your relevant Director, your Manager and the Risk Manager immediately by telephone. Incident report must be completed. Incident must be added to service risk register. |